SELF CLOSING TAP FOR SHOWER

single tap with automatic time delayed shut-off

Inlet 3/4 ''

Flow rate 8L/mn at 417 kPa

Flow duration from 20 to 35 sec self cleaning mechanism with ruby,

anti-corrosion and anti-liming materials,

For concealed installation.