PRESTO NEO® DUO S® – 64766

Single wall-mounted self-closing tap, with an ergonomic soft touch lever to control water flow, 1/2", hot or cold water. As PRESTO NEO® DUO S® antiblocking system ref.64766 or technically equivalent. Equipped against corrosion and lime scale with a self-cleaning needle and ruby mechanism, 1.9l/mn with flow regulator. Dual timed-flow: 7 sec. (lever upwards) and 11 sec. (lever downwards). Flow rate and flow time could be changed by internal device only. With S® antiblocking system to prevent blocking in continuous flow.

Or

Single wall-mounted self-closing tap, with an ergonomic soft touch lever to control water flow, 1/2", hot or cold water. As PRESTO NEO® DUO S® antiblocking system ref.64766 or technically equivalent. Including:

- A self-cleaning needle and ruby mechanism

- 1.9 l/mn with flow regulator

- Dual timed-flow: 7 sec. (lever upwards) and 11 sec. (lever downwards)

- Flow rate and flow time could be changed by internal device only

- Soft touch

- With S® antiblocking system to prevent blocking in continuous flow

.